Features

◇Fast response time

◇High photo sensitivity

◇Small junction capacitance

◇Pb free

Descriptions

5003MDis a high speed and high sensitive PIN

photodiode in a standard 5Φplastic package.

The device is matched to infrared emitting diode.

Applications

◇ Infrared applied system.

◇ Counters and sorters

◇ Encoders

◇ Floppy disk drive.

◇ Optoelectronic switch

◇ Video camera, Tape and card readers

◇ Position sensors

Device Selection Guide

| LED Part No. | Chip | Lens Color |
| --- | --- | --- |
|  | Material | Black |
| F5接受管 | Silicon |  |

Package Dimensions

Notes:

1.All dimensions are in millimeters

2.Tolerances unless dimensions ±0.1mm

Absolute Maximum Rating (Ta=25℃)

| ## Parameter | Symbol | Absolute Maximum Rating | Unit |
| --- | --- | --- | --- |
| Reverse Voltage | VR | 30 | V |
| Power Dissipation | PD | 75 | mW |
| Operating Temperature | Topr | -25～+85 | ℃ |
| Storage Temperature | Tstg | -40～+85 | ℃ |
| Soldering Heat (5s) | Tsol | 260 | ℃ |

Notes: *1:Soldering time≦5 seconds.

Electro-Optical Characteristics (Ta=25℃)

| Parameter | Symbol | Min. | TYP. | Max. | Unit | Condition |
| --- | --- | --- | --- | --- | --- | --- |
| Rang Of Spectral Bandwidth | λ0.5 | 400 | --- | 1100 | nm | --- |
| Wavelength Of Peak Sensitivity | λP | --- | 940 | --- | V | --- |
| Open-Circuit Voltage | VOC | --- | 0.39 | --- | V | Ee=5mW/cm2 λp=940nm |
| Short- Circuit Current | ISC | --- | 35 | --- | μA | Ee=1mW/cm2 λp=940nm |
| Reverse Light Current | IL | 25 | 35 | --- | μA | Ee=1mW/cm2 λp=940nm VR=5V |
| Reverse Dark Current | ID | --- | 5 | 30 | nA | Ee=0mW/cm2 VR=10V |
| Reverse Breakdown Voltage | BVR | 32 | 170 | --- | V | Ee=0mW/cm2 IR=100μA |
| Total Capacitance | Ct | --- | 18 | --- | pF | Ee=0mW/cm2 VR=5V f=1MHz |
| Rise/Fall Time | tr/tf | --- | 45/45 | --- | nS | VR=10V RL=1000Ω |

Note:

- Luminous intensity is measured with a light sensor and filter combination that approximates the CIE eye-response curve.

- θ1/2 is the off-axis angle at which the luminous intensity is half the axial luminous intensity.