Mbits-3-Axis Digital Accelerometer

Description

This is a high resolution digital accelerometer on Mbits.

The module agile enough to detect single and double taps. It's ideal for motion detection, Gesture detection as well as robotics.

Usage

Here we show how to read the raw data from the accelerometer.

- Connect the data cable to the communication port of Mbits.

- Open the software and upload the following program to Mbits.

- ① Letscode Mode

- ② Code Mode

- // Language ArduinoC

- #include <MPU6050_tockn.h>

- #include <Wire.h>

- MPU6050 mpu6050(Wire);

- void setup() {

- Serial.begin(115200);

- Wire.begin(22,21);

- mpu6050.begin();

- mpu6050.calcGyroOffsets(true);

- }

- void loop() {

- mpu6050.update();

- Serial.println(String("x的值：") + String((mpu6050.getAccX())));

- Serial.println(String("y的值：") + String((mpu6050.getAccY())));

- Serial.println(String("z的值：") + String((mpu6050.getAccZ())));

- delay(1000);

- }

- After the upload is successful,You can observe the data through the serial monitor.