Mbits-Button

Description

There are two buttons on the Mbits, called button_A and button_B.

Usage

The following sketch demonstrates a simple application of the module.

- Connect the data cable to the communication port of Mbits.

- Open the software and upload the following program to Mbits.

- ① Letscode Mode

- ② Code Mode

- // Language ArduinoC

- #include <FastLED.h>

- #include "Dots5x5font.h"

- #define NUM_ROWS 5

- #define NUM_COLUMNS 5

- #define NUM_LEDS (NUM_ROWS * NUM_COLUMNS)

- #define LED_PIN 13

- #define LED_TYPE WS2812B

- #define COLOR_ORDER GRB

- #include <Button.h>

- CRGBArray<NUM_LEDS> leds;

- uint8_t max_bright =10;

- CRGB myRGBcolor_r32m(255, 0, 0);

- const uint8_t maxBitmap_r32m[]= {

- B11111,B11111,B11111,B11111,B11111

- };

- CRGB myRGBcolor_rzbc(0, 255, 93);

- const uint8_t maxBitmap_rzbc[]= {

- B11111,B11111,B11111,B11111,B11111

- };

- void Display(const byte dat[],CRGB myRGBcolor){

- for (int c = 0; c < 5; c++){

- for (int r = 0; r < 5; r++){

- if (bitRead(dat[c], r)){

- leds[c * 5 + 4-r] = myRGBcolor;

- }

- FastLED.show();

- delay(5);

- }

- }

- }

- void setup() {

- FastLED.addLeds<LED_TYPE, LED_PIN, COLOR_ORDER>(leds, NUM_LEDS);

- FastLED.setBrightness(max_bright);

- }

- void loop() {

- if (buttonA.isPressed() && !buttonB.isPressed()) {

- Display(maxBitmap_r32m,myRGBcolor_r32m);

- }

- if (buttonB.isPressed() && !buttonA.isPressed()) {

- Display(maxBitmap_rzbc,myRGBcolor_rzbc);

- }

- if (buttonAB.isPressed()) {

- FastLED.clear();

- }

- }

- After the upload is successful, you can see When the button a is pressed, the matrix will be red, when the button B is pressed, the matrix will be green, and when the buttons a and B are pressed at the same time, the matrix will not be displayed.