Mbits-Microphone

Description

This is the microphone module on Mbits, which is a low-power, omnidirectional microphone with an analog output.

Normally, the analog value of the microphone varies from 1800 to 1900. When sound is detected, the analog value will decrease.

Usage

Here we show a simple application of the module.

- Connect the data cable to the communication port of Mbits.

- Open the software and upload the following program to Mbits.

- ① Letscode Mode

- ② Code Mode

- // Language ArduinoC

- #include <FastLED.h>

- #include "Dots5x5font.h"

- #define NUM_ROWS 5

- #define NUM_COLUMNS 5

- #define NUM_LEDS (NUM_ROWS * NUM_COLUMNS)

- #define LED_PIN 13

- #define LED_TYPE WS2812B

- #define COLOR_ORDER GRB

- CRGBArray<NUM_LEDS> leds;

- uint8_t max_bright =10;

- CRGB myRGBcolor_zyvx(255, 0, 0);

- const uint8_t maxBitmap_zyvx[]= {

- B00000,B11011,B00000,B10001,B01110

- };

- CRGB myRGBcolor_x6aa(16, 255, 0);

- const uint8_t maxBitmap_x6aa[]= {

- B00000,B11011,B00000,B10001,B01110

- };

- void Display(const byte dat[],CRGB myRGBcolor){

- for (int c = 0; c < 5; c++){

- for (int r = 0; r < 5; r++){

- if (bitRead(dat[c], r)){

- leds[c * 5 + 4-r] = myRGBcolor;

- }

- FastLED.show();

- delay(5);

- }

- }

- }

- void setup() {

- FastLED.addLeds<LED_TYPE, LED_PIN, COLOR_ORDER>(leds, NUM_LEDS);

- FastLED.setBrightness(max_bright);

- }

- void loop() {

- if ((analogRead(35)) < 1700) {

- Display(maxBitmap_zyvx,myRGBcolor_zyvx);

- } else {

- Display(maxBitmap_x6aa,myRGBcolor_x6aa);

- }

- }

- After the upload is successful, You can see that the matrix is normally green and briefly red after high fives.