Mbits-RGB Matrix

Description

The RGB Matrix module is an 5*5 matrix display module composed of 25 rgb led lamp beads. It can respond to changes in input signals in monochrome, full-color, gradual, and horse racing modes, and can also display simple graphics.

Usage

The following sketch demonstrates several simple application of the module.

- Text display

- Connect the data cable to the communication port of Mbits.

- Open the software and upload the following program to Mbits.

- ① Letscode Mode

- ② Code Mode

- // Language ArduinoC

- #include <FastLED.h>

- #include "Dots5x5font.h"

- #define NUM_ROWS 5

- #define NUM_COLUMNS 5

- #define NUM_LEDS (NUM_ROWS * NUM_COLUMNS)

- #define LED_PIN 13

- #define LED_TYPE WS2812B

- #define COLOR_ORDER GRB

- CRGBArray<NUM_LEDS> leds;

- uint8_t max_bright =10;

- CRGB myRGBcolor_adx5(255, 0, 0);

- void plotMatrixChar(CRGB (*matrix)[5], CRGB myRGBcolor, int x, char character, int width, int height){

- int y = 0;

- if (width > 0 && height > 0){

- int charIndex = (int)character - 32;

- int xBitsToProcess = width;

- for (int i = 0; i < height; i++){

- byte fontLine = FontData[charIndex][i];

- for (int bitCount = 0; bitCount < xBitsToProcess; bitCount++){

- CRGB pixelColour = CRGB(0, 0, 0);

- if (fontLine & 0b10000000){

- pixelColour = myRGBcolor;

- }

- fontLine = fontLine << 1;

- int xpos = x + bitCount;

- int ypos = y + i;

- if (xpos < 0 || xpos > 10 || ypos < 0 || ypos > 5);

- else{

- matrix[xpos][ypos] = pixelColour;

- }

- }

- }

- }

- }

- void ShowString(String sMessage){

- CRGB matrixBackColor[10][5];

- int mapLED[] = {0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23,24};

- int messageLength = sMessage.length();

- for (int x = 0; x < messageLength; x++){

- char myChar = sMessage[x];

- plotMatrixChar(matrixBackColor, myRGBcolor_adx5, 0 , myChar, 5, 5);

- for (int sft = 0; sft <= 5; sft++){

- for (int x = 0; x < NUM_COLUMNS; x++){

- for (int y = 0; y < 5; y++){

- int stripIdx = mapLED[y * 5 + x];

- if (x + sft < 5){

- leds[stripIdx] = matrixBackColor[x + sft][y];

- }else{

- leds[stripIdx] = CRGB(0, 0, 0);

- }

- }

- }

- FastLED.show();

- if (sft == 0){

- FastLED.delay(200);

- }else{

- FastLED.delay(30);

- }

- }

- }

- }

- void setup() {

- FastLED.addLeds<LED_TYPE, LED_PIN, COLOR_ORDER>(leds, NUM_LEDS);

- FastLED.setBrightness(max_bright);

- }

- void loop() {

- ShowString("Letscode");

- }

- After the upload is successful, you can see several letters of “letscode” are displayed quickly.

- DIY Images

- Connect the data cable to the communication port of Mbits.

- Open the software and upload the following program to Mbits.

- ① Letscode Mode

- ② Code Mode

- // Language ArduinoC

- #include <FastLED.h>

- #include "Dots5x5font.h"

- #define NUM_ROWS 5

- #define NUM_COLUMNS 5

- #define NUM_LEDS (NUM_ROWS * NUM_COLUMNS)

- #define LED_PIN 13

- #define LED_TYPE WS2812B

- #define COLOR_ORDER GRB

- CRGBArray<NUM_LEDS> leds;

- uint8_t max_bright =10;

- CRGB myRGBcolor_uh2h(255, 0, 0);

- const uint8_t maxBitmap_uh2h[]= {

- B00100,B01110,B11111,B10001,B11111

- };

- void Display(const byte dat[],CRGB myRGBcolor){

- for (int c = 0; c < 5; c++){

- for (int r = 0; r < 5; r++){

- if (bitRead(dat[c], r)){

- leds[c * 5 + 4-r] = myRGBcolor;

- }

- FastLED.show();

- delay(5);

- }

- }

- }

- void setup() {

- FastLED.addLeds<LED_TYPE, LED_PIN, COLOR_ORDER>(leds, NUM_LEDS);

- FastLED.setBrightness(max_bright);

- }

- void loop() {

- Display(maxBitmap_uh2h,myRGBcolor_uh2h);

- }

- After the upload is successful, you can see the red windmill pattern in the center of the matrix.You can also change red to other colors.

- Dynamic Image

- Connect the data cable to the communication port of Mbits.

- Open the software and upload the following program to Mbits.

- ① Letscode Mode

- ② Code Mode

- // Language ArduinoC

- #include <FastLED.h>

- #include "Dots5x5font.h"

- #define NUM_ROWS 5

- #define NUM_COLUMNS 5

- #define NUM_LEDS (NUM_ROWS * NUM_COLUMNS)

- #define LED_PIN 13

- #define LED_TYPE WS2812B

- #define COLOR_ORDER GRB

- CRGBArray<NUM_LEDS> leds;

- uint8_t max_bright =10;

- CRGB myRGBcolor_g00e(255, 0, 0);

- const uint8_t maxBitmap_g00e[]= {

- B10000,B10000,B10000,B10000,B10000

- };

- CRGB myRGBcolor_4m3g(250, 255, 0);

- const uint8_t maxBitmap_4m3g[]= {

- B01000,B01000,B01000,B01000,B01000

- };

- CRGB myRGBcolor_kqbq(0, 119, 255);

- const uint8_t maxBitmap_kqbq[]= {

- B00100,B00100,B00100,B00100,B00100

- };

- CRGB myRGBcolor_a8c4(0, 255, 119);

- const uint8_t maxBitmap_a8c4[]= {

- B00010,B00010,B00010,B00010,B00010

- };

- CRGB myRGBcolor_e6v8(199, 0, 255);

- const uint8_t maxBitmap_e6v8[]= {

- B00001,B00001,B00001,B00001,B00001

- };

- void Display(const byte dat[],CRGB myRGBcolor){

- for (int c = 0; c < 5; c++){

- for (int r = 0; r < 5; r++){

- if (bitRead(dat[c], r)){

- leds[c * 5 + 4-r] = myRGBcolor;

- }

- FastLED.show();

- delay(5);

- }

- }

- }

- void setup() {

- FastLED.addLeds<LED_TYPE, LED_PIN, COLOR_ORDER>(leds, NUM_LEDS);

- FastLED.setBrightness(max_bright);

- }

- void loop() {

- Display(maxBitmap_g00e,myRGBcolor_g00e);

- delay(500);

- Display(maxBitmap_4m3g,myRGBcolor_4m3g);

- delay(500);

- Display(maxBitmap_kqbq,myRGBcolor_kqbq);

- delay(500);

- Display(maxBitmap_a8c4,myRGBcolor_a8c4);

- delay(500);

- Display(maxBitmap_e6v8,myRGBcolor_e6v8);

- delay(500);

- FastLED.clear();

- }

- After the upload is successful, you can see the center of the matrix from left to right showing red, yellow, blue, green and purple alternately.